Chapter 14

Monitoring and Metering

Management is that for which there is no algorithm. Where there is

an algorithm, it’s administration.

– ROGER NEEDHAM

The market is not an invention of capitalism. It has existed for

centuries. It is an invention of civilization.

– MIKHAIL GORBACHEV

14.1	Introduction

In addition to the burglar alarms we discussed in the last chapter, your home will
	likely have a number of other monitoring devices: utility meters, baby monitors,
	smoke detectors, exercise equipment, health trackers and connected appliances.
	You may also buy value online for some metering systems, from a prepayment
	utility meter in your home through prepaid postage labels. An increasing num-
	ber of systems are concerned with monitoring and metering human activities
	and indeed the natural environment too.	They go back a long way.	James

Watt, the steam engine pioneer, didn’t just sell engines; he licensed his patents
	using a sealed counter that measured the number of revolutions an engine had
	made. His inspectors read these from time to time and billed the customer for
	royalties.

Electronic systems that use cryptography and tamper-resistance have dis-

placed most of the older mechanical systems and opened up all sorts of new
	applications. Ticketing is huge, from transport tickets through sports events to
	coupons; my case study for ticketing is the prepayment meters used for gas and
	electricity. Then I’ll turn to vehicle systems. The most familiar of these may be
	taxi meters but as these are being replaced by phone apps, I’ll mainly discuss
	tachographs – devices used in Europe and Australia to record the speed and
	working hours of truck and coach drivers, and in the USA to record the comings
	and goings of bank trucks. My third case study is the curfew tags used in the
	USA to monitor criminal suspects before trial and in the UK for parolees after
	release. My fourth is the electronic postage meters used to frank letters and

457

14.2. PREPAYMENT METERS

packages.

Many of these new applications follow the traditional IT industry mantra

of ‘ship it Tuesday and get it right by version 3’. We do have the beginnings
	of general standards for IoT security, such as the draft ETSI standard EN 303
	645 which lays out the usual motherhood-and-apple-pie stu↵ like no default
	passwords, protecting crypto keys, updateable software, minimising the attack
	surface and allowing users to delete personal information [640]. But turning
	basic principles into good engineering takes e↵ort, and we can learn a lot from
	applications that have already gone through at least one iteration of attack and
	defence. I hope the case studies in this chapter will give some of the needed
	contextual insight.

You’ll recall that in order to defeat a burglar alarm it is su�cient to make

it appear unreliable. Meters add further subtleties.

When we discussed an alarm in a bank vault, we were largely concerned

with attacks on communications (though sensor defeats also matter). But many
	metering systems are much more exposed physically. A taxi driver may want
	the meter to read more miles or more minutes than were actually worked, so
	may manipulate it into over-measuring. With tachographs, it’s the reverse: the
	truck driver usually wants to drive above the speed limit, or work dangerously
	long hours, so wants the tachograph to ignore some driving. Utility consumers
	may want their meters to ignore some of the passing electricity or gas. Criminal
	defendants and parolees may want to evade a curfew order. In such cases, the
	subject of surveillance may cause the device to make false readings, or simply
	to fail. There are also underground markets for exploits of various kinds.

Many metering and monitoring systems are also concerned with evidence.

An opponent could get an advantage either by manipulating communications
	(such as by replaying old messages) or by falsely claiming that someone else
	had done so. With postal franking systems, it’s not su�cient for the attacker to
	cause a failure (as then he can’t post his letters). And we need to understand
	the real threats. The post o�ce is mostly concerned with stopping wholesale
	fraud, such as crooked direct marketers who bribe postal employees to slip a
	truckload of mail into the system. The system may look like it’s designed to
	stop external fraud, but its real focus is internal.

14.2	Prepayment Meters

There are many systems where the user pays in one place for a token – whether
	a magic number, or a cardboard ticket with a magnetic strip, or an app that
	displays a QR code, or even a rechargeable chip card – and uses the stored
	value somewhere else. Examples include transport tickets, photocopier cards
	in libraries, lift passes at ski resorts, and washing-machine tokens in university
	halls of residence.

The main protection goal is usually to prevent the tokens being forged at

scale. Duplicating a single ticket is not too hard, and repeating a magic number
	is easy. Such scams can be prevented if we make all the tokens unique and all
	the devices online. But that makes things fragile; if people can’t get on the bus

| Security Engineering | 458 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

in a mobile network black spot, or can’t use a ski lift or a washing machine if a
	data centre is down, that can damage the business and cost real money. So the
	replay and forgery detection must sometimes be done o✏ine. But if we simply
	encipher all our tokens using a universal master key, a villain could extract it
	from a stolen terminal and set up in business selling tokens.	What are our

options?

In most ticketing systems, procedural fraud is easy. A free rider can jump the

barrier at a subway station; an electricity meter can have a bypass switch wired
	across it. But most people won’t cheat unless someone makes it seem easy and
	safe by industrialising it. To maximise revenue, petty fraud should be at least
	slightly inconvenient and – more importantly – there should be mechanisms to
	prevent anyone forging tickets at scale.

The ﬁrst example I’ll discuss is the prepayment electricity meter. I chose

this because I was lucky enough to consult on a project to electrify three million
	households in South Africa (a central election pledge made by Nelson Mandela
	when he took power). This work is described in some detail in [93]. By December
	2019, the STS speciﬁcation we developed was used in 68 million meters in 98
	countries. Most of the lessons learned apply directly to other ticketing systems.

14.2.1	Utility metering

Householders who can’t get credit buy gas and electricity services using pre-
	payment meters (Figure 14.1). In the old days they were coin-operated, but
	the costs of coin collection led vendors to develop token-based meters instead.
	This technology was driven by less developed countries, and most notably by
	South Africa, where it became a national priority to electrify the townships; as
	many of the houses were informally constructed, and the owners did not even
	have addresses (let alone credit ratings), prepayment was the only way to go.
	Over 2 million meters were installed during Nelson Mandela’s term of o�ce as
	President, and there are now an estimated 10 million in use there. The largest
	installation is 35 million in Indonesia, and they are common in in Africa, Asia
	and Latin America, as well as in some developed countries; most meters in
	Northern Ireland are prepayment. The typical developed country might have
	about 10% of households using prepayment meters, because they’re on welfare
	or have court judgments against them.

The customer goes to a shop and buys a token, which can be a card, a

cardboard ticket with a magnetic strip, or a 20-digit magic number. Most of
	South Africa’s meters use a magic number. This is convenient for the customer,
	as a ticket can be bought at a supermarket checkout, at an ATM, over the phone
	or online.

The token is really just one or more instructions, encrypted using a key

unique to the meter, and saying something like ‘meter 12345 – dispense 50KWh
	of electricity!’ The meter interrupts the supply when the credit runs out. Some
	tokens have engineering functions too. Special tokens may be used to change
	prices: if the power company charges di↵erent rates for the daytime and evening,
	the meter may need updates on the relative prices and the times at which the
	tari↵s change.

| Security Engineering | 459 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

Of the UK’s electricity meters, about twice as many use smartcards as mag-

netic tickets. The former do not use the STS standard but are able to report
	consumption patterns, tampering attempts and so on back to the power com-
	pany. The magnetic-ticket and magic-number meters do not have such a back
	channel.	There is currently a project to replace all the meters in most EU

countries with smart meters which report readings and other data over a radio
	link, and which can be set remotely into prepayment mode. Smart meters have
	already been installed in other countries with mixed results. I’ll return to them
	later.

Figure 14.1: – a prepayment electricity meter (Courtesy of Schlumberger)

Prepayment was the only way that less developed countries could electrify

millions of homes quickly. In the developed world, the main incentive was re-
	ducing bad debts and other administrative costs. An added beneﬁt is energy
	saving. In areas where most meters are prepaid, electricity consumption is up
	to 10% lower, as its cost becomes more salient to the householder.

14.2.2	How the system works

The security requirements for prepayment meters seem straightforward. Tokens
	should not be easy to forge, while genuine tokens should not work in the wrong
	meter, or in the right meter twice. The usual strategy is to tie each token to a
	unique meter, so that someone can’t use the same magic number in two di↵er-
	ent meters, and also make each token unique using serial numbers or random
	numbers, so that the same token can’t be used twice in the same meter. But it
	took a surprising amount of experience to develop this simple idea into a robust
	system.

Each meter has a crypto key to authenticate its instructions from the vending

| Security Engineering | 460 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

machine. Early systems had one for each neighbourhood, usually in a local store.
	It had a vend key KV which is the master key for a neighborhood, and each
	meter has a device key KID derived by encrypting its meter ID under the vend
	key:

KID = {ID}KV

This is the same key diversiﬁcation technique described for parking lot ac-

cess devices in Chapter 4, and it works ﬁne where all the tokens are bought
	locally. But real life is usually more complicated. In Britain, deregulation of the
	electricity industry led to dozens of electricity companies who buy power from
	generators and sell it onward to households through a common infrastructure,
	so meters change ownership between multiple power companies with di↵erent
	tari↵ structures. In South Africa, many people commute long distances, so they
	want to buy tickets where they work. So we started with protocols to send a
	customer meter key from the vending station that ‘owns’ the meter to another
	station, and to pass sales data in the opposite direction for balancing and settle-
	ment, somewhat like in ATM networks. In 2007 we introduced online vending;
	a central server has a hardware security module with all the vend keys, so a
	customer can buy a magic number over the Internet or via their mobile phone.
	This server sells directly to seven million customers and also via about 10,000
	online vend points such as ATMs and shops.

Statistical balancing is used to detect non-technical losses, that is, theft of

power through meter tampering or unauthorized connections to mains cables.
	We compare the readings on a feeder meter, which might supply 30 houses, with
	token sales to those houses. But customers hoard tickets and meter readers lie
	about when they read the meter, so the discrepancy is a noisy signal. You can
	use it as a source of leads for your investigation team, and as statistical check
	on your bookkeeping systems, but that’s about it.

There were cases where vending machines were stolen and used to sell to-

kens in competition with the utility. Eliminating such a ‘ghost vendor’ generally
	means changing the keys in all the local meters; there are a few stolen machines
	still out there, operated by crime syndicates. The countermeasure was to main-
	tain a credit balance in the vending machine’s security chip that also protects
	vend keys and foreign meter keys. The balance is decremented with each sale
	and only credited again when cash is banked; the operating company then sends
	a magic number that reloads the chip with credit. So we have an accounting
	system enforced by a value counter at the point of sale, rather than by ledger
	data kept on servers at the utility. However, the strategic direction was central-
	isation, to save the e↵ort and expense of managing resellers, and operators have
	replaced o✏ine vending machines by online vending points that get their tokens
	in real time from a central service.

14.2.3	What goes wrong

As with burglar alarms, environmental robustness is critical. Apart from the
	huge range of temperatures (as variable in South Africa as in the continental

| Security Engineering | 461 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

United States) many areas have severe thunderstorms: the meter can be thought
	of as a microprocessor with a 3-kilometer lightning conductor attached.

When meters were destroyed by lightning, the customers complained and got

credit for the value they said was still unused. So their next step was to poke live
	mains wires into the meter to try to emulate the e↵ects of the lightning. One
	make of meter would give unlimited credit if the circuitry under the token slot
	was destroyed, so service denial attacks worked well enough to become popular.

It was to get worse. Kids in Soweto observed that when there was a brown-

out – a fall in voltage from 220 to 180 volts – a particular make of meter went to
	maximum credit. Soon they were throwing steel chains over the 11KV feeders
	and crediting all the meters in the neighborhood. This bug wasn’t picked up
	because brown-out testing hadn’t been speciﬁed. Developed-country environ-
	mental standards were inadequate for use in Africa. The responsible company
	almost went bust after 100,000 meters had to be pulled out and re-ROMmed.

There were numerous other bugs. One make of meter didn’t vend a speciﬁed

quantity of electricity, but so much worth of electricity at such-and-such a rate.
	Vending sta↵ discovered that the tari↵ could be set to a minute amount, and
	the meter would operate almost for ever. Another allowed refunds, but a copy
	of the refunded token could still be used. Another meter remembered only the
	last token serial number entered, so by alternately entering duplicates of two
	tokens it could be charged up indeﬁnitely.

As elsewhere, the real security breaches resulted from bugs and blunders that

were discovered by accident and exploited in quite opportunistic ways. Some of
	the exploits scaled up and cost millions to ﬁx.

Other lessons learned, which we wrote up in [93], were:

• prepayment may be cheap and simple so long as you control the marketing
		convenience stores, banks and supermarkets, it can become expensive,
		complicated and risky;

• if you don’t get the security infrastructure right ﬁrst time, then changing
		at distant shops, to support commuters;

• recycle technology if you can, as it’s likely to have fewer bugs. Much of

• use multiple experts. One expert alone can not usually span all the issues,

• you absolutely need prolonged ﬁeld testing. This is where many errors

The main lesson learned in the years after the initial deployment was to

design out scalable fraud, which meant centralisation. There are still procedural
	exploits; for example, as any company can become a reseller, buying meters
	and a vending station on the market, crooked ﬁrms can set up rogue meters
	in community housing estates and direct the tenants to buy tokens from them

| Security Engineering | 462 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

instead. So prepayment does not entirely abolish the need for good old-fashioned
	audit, energy balancing and inspection – and neither does it entirely solve the
	problems of local corruption or broader state capture in less developed countries.

What we learned ended up in the STS speciﬁcations that are now used by

dozens of manufacturers worldwide. One compromise did come back to bite
	us, though.	The date in the STS meters rolls over in 2024, which was the

distant future back in the early 1990s when we were doing the work1. Now that
	there are 60 million meters in almost 100 countries, it’s going to cost utilities
	hundreds of millions to give each customer a special key-change ticket to manage
	the rollover. (The positive side of the key change is that the remaining ghost
	vending machines will be ﬁnally put out of business.) So, when designing a new
	system, please think of sustainability not just as ‘Will this system be OK for
	the next 30 years?’ but ‘Will this be OK for the next 100 years?’ You may just
	live long enough to be embarrassed!

14.2.4	Smart meters and smart grids

In the early 2000s, the metering industry started selling the idea of a smart me-
	ter – a meter with real-time communications to a central server so that it could
	be read remotely. This had been patented as long ago as the 1970s but was de-
	veloped into a broader concept involving not just billing, and prepayment if need
	be, but ﬁne-grained pricing, power outage reporting and power quality moni-
	toring. Automatic meter reading (AMR) was superseded by advanced metering
	infrastructure (AMI); the latter has two-way communications, so commands can
	be send to the meter remotely. Pricing can be complex, including both time-of-
	day and demand-response tari↵s. The beneﬁts sold to utilities included reduced
	billing costs and easier debt collection. The case made to governments included
	reducing peak demand and thus the number of power stations required. Mar-
	keters talked about ‘smart grids’, talked excitedly of your meter being able to
	control domestic appliances and to negotiate real-time tari↵s with the market.
	A more sober claim was that smart meters would pay for themselves by making
	users more conscious of how much electricity they used, thus saving money. The
	beneﬁt to the meter vendor was replacing a product that cost $15 and lasted
	for 50 years with one that cost at least $50 and lasted for maybe 15.

There are many issues with smart metering. Researchers ﬁrst raised general

privacy concerns about ﬁne-grained consumption data going to utilities; if the
	meter is set to monitor consumption by the minute or even by the second, the
	utility can work out how many people are in the home, when they eat, when they
	shower and when they sleep. This leads to direct concerns around predatory
	marketing, and indirect concerns around third-party access – whether via law-
	enforcement warrants, abuse of authorised access, or the perhaps inevitable
	intrusion of the advertising ecosystem.	This led to debates about the time

granularity of measurement, and how much data should be held in the meter

1We had to ﬁt everything into the 66 bits of a 20-digit token, and although we thought of

having an extra bit in the counter to get an extra 31 years, that would have meant a time
	unit of 2 minutes rather than one, which would have made selling multiple tokens for a meter
	at the same time tricky. But we did have the foresight to provide for resetting the counter on
	key change.

| Security Engineering | 463 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

versus centrally. Then we noticed that putting a remotely commandable o↵
	switch in all of a country’s homes creates a major cyber-war threat; if an enemy
	can switch o↵ your electricity supply they can quickly close down your economy,
	or hold you to ransom [105]. This led to a scramble by the national-security
	agencies. But perhaps the biggest bundle of issues was around the diverging
	incentives of the various stakeholders. Utilities want to sell lots of energy, while
	governments want to save it and to reduce peak demand. So who would win?

The pioneer was Italy, where the utility ENEL started installing smart me-

ters in 2001. Their main concern was power theft, particularly in southern Italy
	where enforcement sta↵ sent to disconnect non-paying customers were threat-
	ened by gangsters. Smart meters enabled defaulters to be switched remotely to
	a prepayment regime. This was seen as a success, and the lobbyists got to work.
	The concept of a smart grid became US policy with the Energy Independence
	and Security Act of 2007 and came forcefully to public attention when Presi-
	dent Obama allocated $4.5bn to its development as the headline measure of the
	American Recovery and Reinvestment Act; the European Parliament followed
	with a 2009 law requiring member states to conduct an economic assessment of
	smart metering by 2012, and if they found it beneﬁcial, mandate its use by 2022
	(with 80% adoption by 2020) [652]. Many countries have now launched national
	or regional smart meter programmes as have a number of U.S utilities, and we
	have some experience of the successes (such as Spain) and failures (including
	the UK and Ontario).

While US utilities tend to be regulated local monopolies, the European model

has competitive generation, regulated transmission and distribution monopolies,
	and competitive retailers. Whether the meters belong to the distribution net-
	work operator or the retailer is a matter of historical accident. It turned out
	that where the distributor owned the meter, replacing all the meters with smart
	meters was straightforward, as a contractor could do a whole street at a time,
	and the meters could be connected to the utility via power-line communications
	with the substation. In Spain, the utilities set up a buyers’ cartel and insisted
	that every supplier’s meter would work with every other supplier’s headend, so
	they got commodity hardware costing under e50 a meter.

However, in countries where the retailer owns the meter, things were not

so simple.	There is a serious problem with incentives: if smart meters are

to pay for themselves by saving energy, then it makes no sense to put them
	under the control of the retailer, which maximises its proﬁt by maximising
	energy sales. Germany did an honest assessment, decided that smart meters
	would be uneconomic, and abandoned the project. Britain unfortunately plowed
	ahead. Its Department of Energy and Climate Change had already had economic
	assessments in 2004, 2007 and 2008 which showed a negative return on the
	investment. Undeterred, they stretched the assumptions about costs, beneﬁts,
	electricity prices and interest rates, came up with a positive assessment in 2009,
	and committed Britain to introducing smart meters not just for electricity but
	for gas too [883].

Outside Europe, the same problems arose where meters were owned by mul-

tiple retailers. New Zealand made smart meters optional, calculating they would
	be worthwhile only for large houses. In Ontario, as in Britain, the government
	pressed on, leading to an expensive failure, documented in the 2014 Annual

| Security Engineering | 464 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

Report of the Auditor General [1197]. The province dealt with 73 local distri-
	bution companies by building a central system to collect all the meter readings
	and making them available to retailers, as well as regulators. The goal of the
	system, to cut peak demand, was not realised at all; the peak-to-trough price
	variation that politicians were prepared to tolerate was not enough to change
	behaviour. The Ontario cost-beneﬁt analysis that had been prepared in 2005 (a
	year after ministers announced the project) turned out to have overestimated
	beneﬁts as $600m when they were at most $88m, while costs ballooned to $2bn;
	the largest retailer spent over $500 per meter on the devices and the systems to
	support them. Overall, energy planning was so poor that the province ended up
	selling surplus power to the USA, subsidising utilities in Michigan and in New
	York State to the tune of billions of dollars.

In Britain, smart metering has evolved into what may be the largest ever

civilian project disaster. Successive governments (Labour, coalition and Conser-
	vative) committed to rolling out smart meters by 2020 as nobody wanted to be
	accused of not being ‘green’. To my way of thinking, wasting £20 billion without
	saving any energy, and displacing better projects that could have yielded real
	savings, was about as un-green as you can get. The project was gold-plated at
	every level, with each home having up to four devices: smart meters for gas and
	electricity, a home hub to connect them to a wireless network, and an in-home
	display so the bill payer could track consumption. (The project started in 2009
	just as people started to use smartphones but was too rigid to switch to using
	apps instead.) Ministers followed the Ontario route of a central meter reading
	server but nonetheless a UK householder who accepts a smart meter from one
	vendor and then moves to a di↵erent supplier to save money usually has to sub-
	mit manual readings thereafter. It took years to agree a national standard for a
	second-generation meter and most of the deployed meter ﬂeet consists of older
	incompatible models; the vendors fought for years to get their own patents in
	there and the o�cials didn’t have the technical knowledge or political support
	to bang heads together. Security mechanisms were retroﬁtted in a panic in the
	mid-2010s once we pointed out that a hostile state could simply turn o↵ British
	households’ power at a time of tension [105]. Whistleblowers who threatened
	to expose the project’s failure, and a likely cost increase from £11bn to £23bn,
	were threatened with prison [919]. The National Audit O�ce then reported at
	the end of 2018 that the project was falling materially short of expectations:
	the plan had been to replace 80% of UK meters by the end of 2020, but only
	12.5m had been done, with 39m yet to do [1391]. What’s more, 70% of the
	meters lost functionality when customers switched supplier (as you have to do
	annually to get decent prices). If government follows its declared strategy of
	moving everyone to second-generation meters, all these old ones will have to be
	replaced; according to a report from November 2019, only 2.3m of the meters
	were the new ones. Cost savings are unlikely as the industry will have to support
	good old-fashioned meters, several types of obsolete smart meter and the new
	smart meters through the 2020s. As for energy savings, there’s no sign. (The
	government could save a lot of energy if it used the meters to move everyone
	to prepayment, but that’s not on the agenda, and could have been done with
	much cheaper kit.) Nobody’s using the data for anything but billing. And now
	o�cials just don’t want to know: in the words of the NAO report, “The Depart-
	ment currently has no plans to continue engagement with consumers after the

| Security Engineering | 465 | Ross Anderson |
| --- | --- | --- |

14.2. PREPAYMENT METERS

rollout is complete.”

Two ﬁnal remarks on smart grids. While the meter makers were doing their

big marketing push in the late 2000s, there was breathless talk of meters helping
	to stabilise the grid by creating demand response and improving measurement.
	The scepticism expressed at the time by experienced power engineers has turned
	out to be justiﬁed. Grids have indeed become more fragile as generation capac-
	ity has moved from large spinning machines attached to the core transmission
	network to hundreds of thousands of windmills and solar panels embedded in the
	wider distribution systems. Recent large outages, such as in South Australia on
	28 September 2016 and the UK on 9 August 2019, were cascade failures, caused
	when a local issue (a storm in Australia, and a lightning strike in England)
	caused a rate of change of frequency in excess of the safety limit, causing fur-
	ther loads to be shed, resulting in undervoltage and further load shedding. A
	complicating factor in each case is that now we have a lot of generation capacity
	embedded on people’s rooftops, shedding load doesn’t work as well as it used to.
	The takeaway is not that we need smart meters, even at the substation level,
	but that we need more inertia in the system – which means buying batteries
	or synchronous condensers. We also need to make the rest of the infrastructure
	more tolerant of outages. Much of the political fury in Britain over the 2019
	power cut came from London commuters being stuck in trains for hours. This
	happened because 60 Siemens Desiro class trains tripped at 49Hz when they
	should have tripped at 48.5Hz, and half of them would not restart because of a
	software bug. Getting them going again required a visit from a technician with
	a laptop2.

Demand response was also supposed to help with peak demand reduction.

Nowhere have smart meters helped. Many countries now have capacity mar-
	kets where grid operators can buy extra megawatts, on timescales of seconds to
	minutes, but these operate using dedicated systems. For example, data centre
	operators who have standby diesel generators and have to run them for half an
	hour a month to make sure they still work, are paid to start them when they’re
	needed. In warmer countries, some people get discounts on their electricity bills
	for allowing their air conditioners to be switched o↵ for half an hour during
	demand peaks. Eventually, the chargers for electric cars will contribute to this
	too, once there are enough of them. But the equipment to do this is always
	separate from the main utility meter; no entrepreneur starting a capacity com-
	pany would want to get entangled with the regulated mess that is metering.
	As for the smart meter marketing vision of your home hub negotiating energy
	prices and turning o↵ your cooker or water heater in response to a price surge,
	that is remote from commercial reality. Firms that sell things like cookers and
	heaters are indeed putting CPUs and communications in them, but they talk to
	the ﬁrm’s own servers, not to other devices; and the idea that politicians would
	allow retail prices to surge to match those on capacity markets is na¨ıve. All
	that smart meters have achieved in Britain is to put a few tens of thousands of
	meter readers out of work, at a cost to the bill payer of £20 billion. Ontario
	was the same, but with one less trailing zero.

2UK trains and railway signals are not allowed to do over-the-air software upgrade because

of national security rules, as the railways are considered to be critical national infrastructure.
	This also means that security patches take days to ship. Well done, MI5!

| Security Engineering | 466 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

14.2.5	Ticketing fraud

Transport ticketing is a larger application than utility metering, but I don’t
	know of any serious and publicly available study of the failure modes of train,
	bus and subway tickets. In the case of London, deregulation of the railways led
	to problems with train companies manipulating ticket sales by booking them at
	stations where they got a larger percentage of the takings; if you’re designing a
	system that shares revenue between vendors, you should try to design out the
	incentive for stakeholders to cheat. There was also a scare after the break of
	Mifare Classic described in section 13.2.5; Transport for London scrambled to
	add intrusion-detection systems to detect fraud.

One type of ticketing on which we do have some real fraud data is the airline

variety. During the 2010s, there emerged an ecosystem of fraudulently obtained
	air tickets and of channels for reselling them. The tickets are obtained by a
	variety of methods, ranging from compromised credit cards through dishonest
	sta↵ at airlines and travel agencies through stolen air miles and hacked book-
	ing systems; the marketing channels include spam, a�liate marketing, sales to
	migrant communities and sales to human tra�ckers. This is all documented
	by Alice Hutchings [936, 937]. The key factors are that plane tickets, unlike
	subway tickets, are su�ciently valuable for such an ecosystem to develop; and
	that while some of the customers know they’re getting bogus tickets, enough of
	them are simply suckers, so you can’t just arrest everyone who turns up for a
	ﬂight with an invalid ticket.

I’ll now look at a class of applications where the attacks are more severe and

prolonged than on electricity meters. The threat model includes sensor manipu-
	lation, service denial, accounting ﬁddles, procedural defeats and the corruption
	of operating sta↵. This exemplary ﬁeld of study is vehicle monitoring systems.

14.3	Taxi meters, tachographs and truck speed
		limiters

A number of systems are designed to monitor and control vehicles. The most fa-
	miliar is probably the odometer in your car. When buying a used car you’ll worry
	whether the car has been clocked, that is, had its indicated mileage reduced. As
	odometers became digital, clocking became a kind of computer fraud [391]. A
	related problem is chipping, that is, replacing or reprogramming the engine con-
	troller. This can be done for two basic reasons. First, the engine controller acts
	as the server for the remote key-entry systems that protect most modern cars
	from theft, as described in Chapter 4; so if you want to steal a car without steal-
	ing the key, you might replace the controller in the street, or else tow the car
	and replace or reprogram the controller later. Second, people reprogram their
	cars’ engine controllers to make them go faster, and the manufacturers dislike
	this because of the increased warranty claims from burned-out engines. So they
	try to make the controllers more tamper resistant, or at least tamper-evident.
	This arms race is described in [624].

Many vehicles now keep logs that are uploaded to the manufacturer during

| Security Engineering | 467 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

servicing. General Motors started equipping some vehicles with black boxes to
	record crash data in 1990. By the time the logging became public in 1999, some
	six million vehicles had been instrumented, and the disclosure caused protests
	from privacy activists [1938]. Indeed, there’s a whole conference, ESCAR, de-
	voted to electronic security in cars. Vehicle security is becoming a hot topic
	again in 2019 because of the growing interest in autonomous operation3.

Other vehicle monitoring systems are ﬁtted after manufacture, and the most

familiar may be the taxi meter. A taxi driver has an incentive to manipulate the
	meter to show more miles travelled (or minutes waited) if he can get away with
	it. There are various other kinds of ‘black box’ used to record the movement of
	vehicles from aircraft through ﬁshing vessels to armored bank trucks, and their
	operators have di↵ering levels of motive for tampering with them. Insurers who
	sell ‘pay-as-you-drive’ insurance to young and high-risk drivers demand that
	they ﬁt black boxes with satellite navigation devices that let the insurer charge
	a couple of pennies a mile for driving along a country road in the afternoon but
	a couple of dollars a mile for evening driving in an inner city [1909]. Any young
	man who wants to impress a lady by driving around town on a Saturday night
	will have an incentive to beat the black box.

14.3.1	The tachograph

The case study I’m going to use here is the tachograph. These devices are used
	to monitor truck drivers’ speed and working hours; in Europe, the traditional
	analogue devices were replaced by digital ones from 2006, and as a truck lasts
	about ten years, most of the ﬂeet is now digital. This gives us some interesting
	data on how such equipment works, and can fail; and it’s an example of how a
	move to digital technology didn’t make things better. What was actually needed
	wasn’t whizzy technology but more enforcement.

Vehicle accidents resulting from a driver falling asleep at the wheel cause

several times more accidents than drunkenness (20 percent versus 3 percent of
	accidents in the UK, for example). Accidents involving trucks are more likely
	to lead to fatal injuries because of the truck’s mass. So most countries regulate
	truck drivers’ working hours. While these laws are enforced in the USA using
	weigh stations and drivers’ log books, countries in Europe use tachographs that
	record a 24-hour history of the vehicle’s speed. Until 2005–6, this was recorded
	on a circular waxed paper chart (Figure 14.2); since then, digital tachographs
	have been introduced and the old system has been largely phased out4.

First let’s look at the old analogue system as our baseline; it’s still in use by

old trucks and buses on Europe’s roads.

The analogue system uses a waxed paper chart that is loaded into the tacho-

graph, which is part of the vehicle’s speedometer/odometer unit. It turns slowly
	on a turntable inside the instrument that turns once every 24 hours, and a speed
	history is inscribed by a ﬁne stylus connected to the speedometer. With some

3Full disclosure: one of my research students is funded by Bosch.
	4Vehicles registered since August 2004 in the UK had to have digital systems ﬁtted, driver

cards have been issued since June 2005 and the use of digital systems in new vehicles became
	mandatory in August 2006; the dates vary slightly for other EU countries.

| Security Engineering | 468 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

Figure 14.2: – a tachograph chart

exceptions that needn’t concern us, it is an o↵ence to drive a truck in Europe
	unless you have a tachograph; if it’s analogue you must have a chart installed,
	and have written on it your starting time and location. You must also keep sev-
	eral days’ charts with you to establish that you’ve complied with the relevant
	driving hours regulations (typically 8.5 hours per day with rules for rest breaks
	per day and rest days per week). If it’s digital you have to have a driver card
	plugged into it; the card and the vehicle unit both keep records.

European law also restricts trucks to 100 km/h (62 mph) on freeways and

less on other roads. This is enforced not just by police speed traps and the
	tachograph record, but directly by a speed limiter that is also driven by the
	tachograph. Tachograph charts are also used to investigate other o↵ences, such
	as unlicensed waste dumping, and by ﬂeet operators to detect fuel theft. So
	there are lots of reasons why a truck driver might want to ﬁddle his tacho-
	graph. Indeed, it’s a general principle in security engineering that one shouldn’t
	aggregate targets. Forcing a truck driver to defeat his tachograph in order to
	circumvent his speed limiter, and vice versa, was a design error – but one that’s
	now too entrenched to change easily.

Most of what we have to say applies just as well to taxi meters and other

monitoring devices. While the truck driver wants his vehicle to appear to have
	gone less distance, the taxi driver wants the opposite. This has little e↵ect on
	the actual tampering techniques.

14.3.2	What goes wrong

According to a survey of 1060 convictions of drivers and operators done before
	the introduction of the new digital system [64], the o↵ences were distributed as
	follows.

| Security Engineering | 469 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

14.3.2.1	How most tachograph manipulation is done

About 70% of o↵ences that result in conviction did not involve tampering but
	exploited procedural weaknesses.	For example, a company with premises in

Dundee and Southampton should have four drivers in order to operate one
	vehicle per day in each direction, as the distance is about 500 miles and the
	journey takes about 10 hours – which is illegal for a single driver to do every day.
	The standard ﬁddle is to have two drivers who meet at an intermediate point
	such as Penrith, change trucks, and insert new paper charts into the tachographs.
	So the driver who had come from Southampton now returns home with the
	vehicle from Dundee. When stopped and asked for his charts, he shows the
	current chart from Penrith to Southampton, the previous day’s for Southampton
	to Penrith, the day before’s for Penrith to Southampton, and so on. In this way
	he can give the false impression that he spent every other night in Penrith and
	was thus legal. This practice of swapping vehicles halfway through the working
	day is called ghosting. It’s even harder to detect in mainland Europe, where a
	driver might be operating out of a depot in France on Monday, in Belgium on
	Tuesday and in Holland on Wednesday.

Simpler frauds included setting the clock wrongly, pretending that a hitch-

hiker is a relief driver, and recording the start point as a village with a very
	common name – such as ‘Milton’ in England or ‘La Hoya’ in Spain. If stopped,
	the driver can claim he started from a nearby Milton or La Hoya.

Such tricks often involve collusion between the driver and the operator.

When the operator is ordered to produce charts and supporting documents
	such as pay records, weigh station slips and ferry tickets, his o�ce may well
	conveniently burn down. (It’s remarkable how many truck companies operate
	out of cheap wooden sheds at a safe distance from the trucks in their yard.)

14.3.2.2	Tampering with the supply

The next largest category of fraud, amounting to about 20% of the total, in-
	volved tampering with the supply to the tachograph instrument, including in-
	terference with the power and impulse supply, cables and seals.

The earliest tachographs used a rotating wire cable – as did the speedometers

in cars up until the early 1980s – that was hard to ﬁddle with; if you jammed
	the truck’s odometer it was quite likely that you’d shear o↵ the cable. More
	recent analogue tachographs are ‘electronic’, in that they use electric cables
	rather than rotating wire. The input comes from a sensor in the gearbox, which
	sends electrical impulses as the prop shaft rotates. This has made ﬁddling much
	easier! A common attack is to unscrew the sensor about a tenth of an inch,
	which causes the impulses to cease, as if the vehicle were stationary. To prevent
	this, sensors are ﬁxed in place with a wire and lead seal. Fitters are bribed
	to wrap the wire anticlockwise rather than clockwise, which causes it to loosen
	rather than break when the sensor is unscrewed. The fact that seals are issued
	to workshops rather than to individual ﬁtters complicates prosecution.

But most of the ﬁddles are much simpler still. Drivers short out the cable

or replace the tachograph fuse with a blown one. (One manufacturer tried to

| Security Engineering | 470 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

stop this trick by putting the truck’s anti-lock braking system on the same fuse.
	Many drivers preferred to get home sooner than to drive a safe vehicle.) There’s
	evidence of power-supply interruption on the chart in Figure 14.2: around 11
	A.M., there are several places where the speed indicated in the outside trace
	goes suddenly from zero to over 100 km/h. These indicate power interruptions,
	except where there’s also a discontinuity in the distance trace. There, the unit
	was open.

14.3.2.3	Tampering with the instrument

The third category of fraud was tampering with the tachograph unit itself.
	The typical o↵ence in this category is miscalibration, usually done in cahoots
	with the ﬁtter but sometimes by the driver defeating the seal on the device.
	This amounted for some 6% of o↵ences, but declined through the 1990s as the
	introduction of digital communications made it easier to tamper with the cable
	instead.

14.3.2.4	High-tech attacks

The state of the tampering art at the time of the survey was the equipment in
	Figure 14.3. The plastic cylinder on the left of the photo is marked ‘Voltage
	Regulator — Made in Japan’ but is certainly not a voltage regulator. (It appears
	to be made in Italy.) It is spliced into the tachograph cable and controlled by
	the driver using the remote control key fob. A ﬁrst press causes the indicated
	speed to drop by 10%, a second press causes a drop of 20%, a third press causes
	it to fall to zero, and a fourth causes the device to return to proper operation.

This kind of device accounted for under 1% of convictions but its use was

believed to be much more widespread. It’s extremely hard to ﬁnd as it can
	be hidden at many di↵erent places in the truck’s cable harness. Police o�cers
	who stop a speeding truck equipped with such a device, and can’t ﬁnd it, have
	di�culty getting a conviction: the sealed and apparently correctly calibrated
	tachograph contradicts the evidence from their radar or camera.

14.3.3	Digital Tachographs

The countermeasures taken against tachograph manipulation vary by country.
	In Britain, trucks are stopped at the roadside for random checks by vehicle
	inspectors, and suspect trucks may be shadowed across the country.	In the

Netherlands, inspectors prefer to descend on a trucking company and audit their
	delivery documents, drivers’ timesheets, fuel records etc. In Italy, data from the
	toll booths on the freeways are used to prosecute drivers who’ve averaged more
	than the speed limit (you can often see trucks parked just in front of Italian
	toll booths). But drivers can arbitrage between the di↵ering control regimes.
	For example, a truck driver operating between France and Holland can keep his
	documents at a depot in France where the Dutch vehicle inspectors can’t get at
	them. The weakness in the UK system was that when a vehicle inspector stopped
	a truck and found evidence of a violation, this would result in a prosecution some

| Security Engineering | 471 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

Figure 14.3: – a tachograph with an interruptor controlled by the driver using
	a radio key fob. (Courtesy of Hampshire Constabulary, England)

months later in the local magistrate’s court. Foreign drivers often just didn’t
	appear.

So the European Union took the initiative to design a uniﬁed electronic

tachograph system to replace the existing paper-based charts with smartcards.
	Each driver now has a driver card that contains a record of his driving hours
	over the last 28 days. Every vehicle registered since 2006 has a vehicle unit that
	can hold a year’s history. There are also workshop cards used by mechanics
	to calibrate devices, and control cards used by law enforcement o�cers to read
	them out at the roadside. In 1998, I was hired by the UK Department of Trans-
	port to look at the new scheme and try to ﬁgure out what would go wrong.
	After talking to a wide range of people from policemen and vehicle inspectors to
	tachograph vendors and accident investigators, I wrote a report [64]. I revisited
	the ﬁeld in 2007 when writing the second edition of this book; it was simulta-
	neously pleasing and depressing to ﬁnd that I’d mostly predicted the problems
	correctly. However a few interesting new twists also emerged. Finally, in 2020,
	in the third edition, we can take a more mature view.

The main objection raised to the project was that it was not clear how going

digital would help combat the procedural frauds that made up 70% of the total.
	Indeed, our pair of drivers ‘ghosting’ between Dundee and Southampton had
	their lives made even easier. It took fourteen years – more than the lifetime of
	a truck – to change over to the new system and meantime a crooked company
	could run one new digital truck and one old analogue one. Each driver will now
	have one chart and one card, with ﬁve hours a day on each, rather than two
	charts which they might accidentally mix up when stopped. This turned out to
	be well-founded. By 2008, some 20% of the vehicle ﬂeet had digital tachographs
	– somewhat more than would be expected – which suggested that operators

| Security Engineering | 472 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

may have been installing digital devices before they need to as they’re easier to
	ﬁddle. In 2020, drivers have multiple cards.

Another objection was that enforcement would be made harder by the loss

of detailed speed and driving hours information. Back in 1998, the Germans
	had wanted the driver card to be a memory device so it could contain detailed
	records; the French insisted on a smartcard, thanks to lobbying from their smart-
	card industry. So the driver card has limited memory, and can only contain a
	limited number of alarm events.

14.3.3.1	System-level problems

The response to the loss of ﬁne-grained data varies by country. Germany went
	for an infrastructure of ﬂeet management systems that accept digital tachograph
	data, digitized versions of the analog data from the existing paper charts, fuel
	data, delivery data and even payroll, and reconcile them all to provide not just
	management information for the trucking company but surveillance data for the
	police. Britain has something similar, although it’s up to the police to decide
	which companies to inspect; unless they do so, data on driving infringements
	is only available to the employer. There are third-party service ﬁrms who will
	analyse this for companies who are keen on saving time, or just demonstrating
	compliance. Germany has also introduced a system of road pricing for heavy
	goods vehicles that gives further inputs into ﬂeet management.

Britain has a network of automatic number plate reader (ANPR) cameras,

initially installed around London to make IRA bombing attacks harder; after
	the Good Friday agreement in 1997 ended that threat, ANPR was not decom-
	missioned but extended nationwide. That was justiﬁed on the basis of detecting
	car tax evaders, but we then saw ANPR data adduced in more and more prose-
	cutions, for everything from terrorism down to burglary. In the case of drivers’
	hours enforcement, the strategy is to verify a sample of logged journeys against
	the ANPR database; where discrepancies are found, the company’s operations
	are then scrutinised more closely.

However, disagreements about privacy and about national economic interests

hindered EU-wide standardization. It’s up to individual countries whether they
	require truck companies to download and analyze the data from their trucks.
	And even among countries that require this, there’s still arbitrage. For example,
	the German police are much more vigorous at enforcing drivers’ hours regula-
	tions than their Italian counterparts. So, under the old analogue system, an
	Italian driver who normally didn’t bother to put a chart in his machine did so
	while driving over the Alps. Meanwhile, the driver of the German truck going
	the other way took his chart out. The net e↵ect was that all drivers in a given
	country were subject to the same level of enforcement. But if the driving data
	got uploaded from the Italian driver’s card and kept on a PC at a truck com-
	pany in Rome then they were subject to Italian levels of enforcement (or even
	less if the police in Rome didn’t care about accidents in Germany). The ﬁx
	was extraterritoriality; an Italian truck driver stopped in Germany can now be
	prosecuted there if he can’t show satisfactory records of his driving in Italy for
	the week before he crossed the border.

| Security Engineering | 473 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

In the UK, foreign drivers who were stopped and ordered to appear at a

magistrates’ court often didn’t turn up.	The real ﬁx turned out to be not

technological, but legal. In March 2018, Britain changed the law to allow spot
	ﬁnes at the roadside. Previously, o�cers could only issue spot ﬁnes for ongoing
	o↵ences, rather than for o↵ences visible in the truck or driver records. This
	change led to a near-tenfold increase in ﬁnes [924].

14.3.3.2	Other problems

Overall, the move from analogue to digital wasn’t an improvement. While com-
	parative fraud statistics of digital and analogue devices have not been collected,
	the view of o�cials is that while the initial detection of an unrealistic journey
	remains much the same, the sophistication of digital defeat devices makes them
	harder to ﬁnd [1726]. And there are other interesting problems with tachographs
	becoming digital.

First, digital tachographs were the ﬁrst system that caused digital signatures

to turn up in court in large numbers. For years, security researchers have been
	writing academic papers with punchlines like “the judge then raises X to the
	power Y, ﬁnds it’s equal to Z, and sends Bob to jail.” The reality is di↵erent.
	Judges found digital signatures di�cult as they were presented as hexadecimal
	strings on little tickets printed out from vehicle units, with no approved ap-
	paratus for veriﬁcation. The police solved the problem by applying standard
	procedures for “securing” evidence. When they raid a dodgy trucking company,
	they image the PC’s disk drive and take copies on DVDs that are sealed in ev-
	idence bags. One gets given to the defence and one kept for appeal. The paper
	logs documenting the copying are available for Their Worships to inspect, along
	with the printouts from the vehicle units.

Second, many drivers have more than one driver card. This is an o↵ence

everywhere but that doesn’t stop it! Drivers borrow them from friends who
	use them only occasionally – for example because they usually drive trucks
	under 3.5 tonnes. And thanks to EU freedom of movement, drivers can easily
	have more than one address: the Jean Moulin of Toulouse may also be Jean
	Moulin of Antwerp. A database, Tachonet, was set up to try to catch duplicate
	applications across European countries but it doesn’t seem to work very well.
	For example, drivers may forget their middle name in one of their countries of
	residence. From 2018 it was made mandatory for Member States to share data
	with it.

Third, there are new kinds of service-denial attacks (as well as the tradi-

tional ones on gearbox sensors, fuses and so on). A truck driver can destroy his
	smartcard by feeding it with mains electricity (even a truck’s 24 volts will do
	ﬁne). Under the regulations he is allowed to drive for 15 days while waiting for
	a replacement. As static electricity destroys maybe 1% of cards a year anyway,
	it’s hard to prosecute drivers for doing this occasionally.

Fourth, I mentioned that the loss of detailed, redundant data on the tacho-

graph chart makes enforcement harder. In the old analogue days, experienced
	vehicle inspectors had a ‘feel’ for when a chart isn’t right, but the analogue
	trace was replaced by a binary signal saying either that the driver infringed the

| Security Engineering | 474 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

regulations or that he didn’t. This spills over into other enforcement tasks; ana-
	logue charts were often used to collect evidence of illegal toxic waste dumping,
	for example, as the recorded speed history often gave an inspector a good idea
	of the truck’s route.

Next, some of the cards in the system (notably the workshop cards used to set

up the instruments, and the control cards used by police and vehicle inspectors)
	are very powerful.	They can be used to erase evidence of wrongdoing.	For

example, if you use a workshop card to wind back the clock in a vehicle unit
	from 10th July to 8th July, then the entries for July 9th and 10th become
	unreadable. Some countries have therefore gone to great lengths to minimise
	the number of workshop cards that fall into bad hands. In the UK, for example,
	truck mechanics have to pass a criminal records check to get one; yet this isn’t
	foolproof as it’s often companies that get convicted, and the wealthy owners of
	crooked truck-maintenance ﬁrms just set up new ﬁrms. There’s no company
	licensing scheme, and although wrongdoers can be blacklisted from acting as
	directors of licensed ﬁrms, crooks just hide behind nominee directors.

14.3.3.3	The resurrecting duckling, or TOFU

There is one interesting spin-o↵ from the world of tachographs.	In the late

1990s, a European Union regulation decreed that, in order to frustrate the use
	of interruptors of the kind shown in Figure 14.3, all digital tachographs had to
	encrypt the pulse train from the gearbox sensor to the vehicle unit. As both
	of these devices contain a microcontroller, and the data rate is fairly low, this
	shouldn’t in theory have been a problem. But how on earth could we distribute
	the keys? If we just set up a hotline that garages could call, it is likely to be
	abused. There’s a long history of ﬁtters conspiring with truck drivers to defeat
	the system, and of garage sta↵ abusing helplines to get unlocking data for stolen
	cars and even PIN codes for stolen car radios.

One solution is given by the resurrecting duckling security policy model,

more prosaically known as trust on ﬁrst use.	This is named after the fact

that a duckling emerging from its egg will recognize as its mother the ﬁrst
	moving object it sees that makes a sound: this is called imprinting. Similarly,
	a ‘newborn’ vehicle unit, just removed from the shrink wrap, can recognize as
	its owner the ﬁrst gearbox sensor that sends it a secret key. The sensor does
	this on power-up. As soon as this key is received, the vehicle unit is no longer a
	newborn and will stay faithful to the gearbox sensor for the rest of its ‘life’. If
	the sensor fails and has to be replaced, a workshop card can be used to ‘kill’ the
	vehicle unit’s key store and resurrect it as a newborn, whereupon it can imprint
	on the new sensor. Each act of resurrection is indelibly logged in the vehicle
	unit to make abuse harder. (This at least was the theory – the implementation
	fell somewhat short in that in one unit the error code for sensor rekeying is the
	same as the error code for a power outage.)

The resurrecting duckling model of key management was originally developed

to deal with the secure imprinting of a digital thermometer or other piece of
	medical equipment to a doctor’s tablet or a bedside monitor [1819]. It is also
	used in wireless LAN extenders; these devices typically use a protocol suite
	called Homeplug AV to transmit data at 155Mb/s over the domestic power mains

| Security Engineering | 475 | Ross Anderson |
| --- | --- | --- |

14.3. TAXI METERS, TACHOGRAPHS AND TRUCK SPEED LIMITERS

from a unit plugged into your broadband hub to a remote unit on another ﬂoor.
	In order to prevent your neighbours being able to use your wiﬁ, each virtual
	network is encrypted with a key that’s set up the ﬁrst time you plug in an
	extender [1437]. The idea is that when you plug something in, it should just
	work; if it mates with the wrong network, you press a button and try again; in
	extremis you can do the security manually by copying a key o↵ the packaging
	into any enrolled device that has a keyboard. As more and more devices with
	constrained interfaces join the Internet of Things, such protocols will become
	more widespread.

14.3.4	Sensor defeats and third generation devices

However, even if the protocols can be secured, the sensors can still be attacked
	directly. Since digital tachographs started shipping, the folks who brought you
	the interruptor now have a new product: a black box containing electromagnets
	and electronics to simulate a gearbox. The errant truck driver unscrews his
	gearbox sensor and places it in this simulator, which comes with its own cable
	and a sensor that he plugs into his actual gearbox. The system now operates
	as before; on command it will either relay impulses faithfully, or discard them,
	or ﬁlter some of them out. The dodgy pulse-train arrives at the tachograph as
	before, but this time beautifully encrypted using triple-DES. Secure sensing is
	harder than it looks!

This became such a nuisance that the EU passed a law in 2009 speciﬁcally

prohibiting, and requiring Member States to check for, “any device, or devices,
	intended to destroy, suppress, manipulate or alter any data, or which is intended
	to interfere with any part of the electronic data exchange between the component
	parts of recording equipment, or which inhibits or alters the data in such ways
	prior to encryption” [652].	It also upgraded the regulations to require that

vehicles registered from 2012 have the ‘third version tachograph’, which requires
	an extra motion sensor as a countermeasure to sensor defeats

14.3.5	The fourth generation – smart tachographs

In 2014 the regulations were updated to introduce the smart tachograph, which
	is required in vehicles registered for the ﬁrst time as from 15 June 2019, and
	adds:

• better security mechanisms to make fraud more di�cult;

• a satellite navigation system, which will record the truck’s location at the
		that;

• a radio link for a cop at the roadside to read tachograph data when the

By now, the reader might feel a certain cynicism about anything called

‘smart’. The regulations are a further move in the direction of pervasive enforce-
	ment, but stop short of demanding that vehicle units keep detailed GPS history.

| Security Engineering | 476 | Ross Anderson |
| --- | --- | --- |

14.4. CURFEW TAGS: GPS AS POLICEMAN

Privacy law in some countries would make that di�cult; in egregious cases, such
	as toxic waste dumping, the authorities can always subpoena the driver’s mo-
	bile phone history5. Meanwhile, vendors o↵er ﬂeet management systems with
	automatic infringement checking, assuring ﬁrms that this will minimise liability.
	We will have to wait and see how this all works out.

But what might be the practicalities of requiring constant GPS monitoring?

We can get some insight from our next application.

14.4	Curfew tags: GPS as policeman

My third case study of monitoring and metering is the curfew tags that criminal
	suspects and paroled o↵enders wear on their ankles in order to constrain and
	monitor their movements. Introduced in Britain in 1999, they are used to cut the
	prison population. Most o↵enders are released after serving half their sentence
	and spend some of their parole period under curfew, which typically means that
	they must stay at home from 7pm to 7am. They wear a curfew tag on an ankle
	bracelet which communicates with a home monitoring station. Others receive
	community sentences instead of prison, with a curfew. Some 20,000 o↵enders
	may be ‘on tag’ at any one time.

Curfew tags have spread to many other countries too. The more expensive

tags contain GPS chips and report the tag wearer’s location constantly to the
	police. In Britain, these devices are worn by sex o↵enders whose curfews prohibit
	them from going near schools, by persistent o↵enders in some police areas, and
	by terrorism suspects. In France, they’re being introduced in domestic violence
	cases [478]. In the USA, they’re o↵ered to many suspects pre-trial as a condition
	of bail (of whom the most famous may have been Harvey Weinstein). There,
	the issue is that while the Federal government pays for its prisoners’ tags, 90%
	of cases are raised by states and cities, which mostly force the tag wearer to
	pay.	Monitoring is dominated by two companies which typically charge $10

a day, with $350 up front.	(When government pays, they only get $2–3 a

day.) So poor defendants get into debt, or get jailed for nonpayment. This is
	short-sighted, as jail costs about $100 a day. Given that the USA has about a
	million people awaiting trial in jail at any time, this is a policy issue with real
	consequences; the number of tag wearers is over 125,000 and has been rising
	since the First Step Act of 2018. Judges see monitoring orders as cost-free; they
	issue them defensively, steadily widening the scope; two-thirds of tag wearers
	are African-American; and unlike with bail, defendants don’t get their money
	back if acquitted [1074].

In 2013–6, I was involved as an expert witness in a number of curfew-tag

cases. The ﬁrst, in 2013, involved a woman convicted of shoplifting who was
	accused of tampering with her curfew tag as it indicated on several occasions
	that she’d left home in the evening. Analysis of the logs relating to my de-
	fendant’s case showed large numbers of false alarms; some of these had good
	explanations (such as power cuts) but many didn’t. The overall picture was of

5All vehicles under 3.5 tons in Europe are required to have embedded phones for eCall

emergency service; this is unfortunately not mandatory for larger vehicles, no doubt because
	of industry lobbying.

| Security Engineering | 477 | Ross Anderson |
| --- | --- | --- |

14.4. CURFEW TAGS: GPS AS POLICEMAN

an unreliable technology surrounded by chaotic procedures and conﬂicts of inter-
	est. The tagging contractor, Serco, not only supplied the tags and the back-end
	systems, but the call centre and the interface to the court system. What’s more,
	if you break your curfew, it isn’t the public prosecutor that takes you before
	the magistrates, but the contractor – relying on expert evidence from one of its
	subcontractors, who helped design the system. We asked the court for access
	to the tag in the case, plus a set of tagging equipment for testing, the system
	speciﬁcations, false alarm statistics and audit reports. The contractor promptly
	replied that “although we continue to feel that the defendant is in breach of the
	order, our attention has been drawn to a number of factors that would allow me
	to properly discontinue proceedings in the public interest” [83].

Several months later, there was a case involving several men subject to ‘ter-

rorism prevention and investigation measures’ (TPIM) orders. These were a
	measure introduced in 2011 that allows the UK government to impose curfews
	on individuals believed to be a terrorist threat but against whom there is insu�-
	cient evidence to mount a prosecution; they have been controversial on human-
	rights grounds. A number of individuals were served with orders restricting their
	movements and ﬁtted with GPS tags to monitor compliance. These tags tended
	to break o↵ after about six months, whereupon the men were prosecuted for tag
	tampering and imprisoned. As this was covered by a secrecy order, the pattern
	only came to light when it was noticed by a London law ﬁrm that represented
	three of them. Again, the government refused to expose any evidence to expert
	examination, and the three men were acquitted, causing embarrassment to the
	then Home Secretary, Theresa May [?]. A few days later, one of them evaded
	surveillance by donning a niqab in a London mosque and leaving as a woman.
	This caused outrage in the press [1904]. The following month, it turned out that
	the two main UK curfew tag contractors – Serco and G4S – had been defrauding
	the government at scale, by charging tag fees for o↵enders who had been ac-
	quitted or who were in prison, abroad or dead, and that this had been going on
	since the contracts started in 2005. They were stripped of their contracts and
	the matter was referred to the Serious Fraud O�ce [1286]. Serco was eventually
	ﬁned £19.2m in 2019 and ordered to pay £3.7m costs; its accountants, Deloitte,
	were ﬁned £4.2m for audits they conducted of the tagging operation.

The dependability of curfew tags came to trial in 2014 in the case of yet

another terrorism suspect who was being held in immigration detention, accused
	of tag tampering, which he denied. This time the government decided to risk
	a trial. The suspect’s lawyers instructed me and a colleague at our Materials
	Science department as experts. We formed the hypothesis that the stress of
	wearing a heavy tag would lead to a fracture of the strap’s ﬁxing, particularly
	for a devout Muslim who prayed ﬁve times a day. The court duly ordered that
	the two of us, and a Saudi research student at our lab, be ﬁtted with GPS
	tags, and we rigged up accelerometers and strain gauges to monitor the test.
	While the student’s tag survived several days of prayer, my tag broke o↵ when
	I caught it on a radiator at home, and my colleague’s after he wore it while
	playing football. The speciﬁcation called for the tag to withstand a 50kg pull,
	and the operating company (which had taken over the business from G4S but
	still used the same specialist subcontractors) claimed that the material from
	which it was made was not liable to fatigue fractures. The government refused
	however ‘for reasons of commercial conﬁdentiality’ to reveal what this material

| Security Engineering | 478 | Ross Anderson |
| --- | --- | --- |

14.4. CURFEW TAGS: GPS AS POLICEMAN

was. No matter; a test of a sliver from the broken ﬁxing lug revealed that it was
	a polycarbonate that does indeed su↵er fatigue fractures. The court ordered us
	to hand back all our samples ‘to protect the contractor’s intellectual property’
	but did not impose a secrecy order on our expert report, which can be found
	at [85]. This suspect was also eventually freed by the court.

By 2015–6 GPS tags, from a new supplier, were being used by the Kent police

to monitor petty o↵enders. The supplier initially made inaccurate claims about
	GPS accuracy (salesmen don’t like to admit anything is less than perfect), and
	there were a couple of trials. This forced us to study the security and reliability
	of GPS, or more generally GNSS (a term that includes not just the original US
	service but the European Galileo system plus the Russian and Chinese o↵erings).
	In such services, a constellation of satellites each broadcasts a very accurate time
	signal, and a receiver seeing four or more of these can solve for its position and
	time. In practice it takes more than that. First, the signal’s propagation depends
	on conditions in the ionosphere, which are variable, leading to error unless this
	can be calibrated against reference stations – a technique called augmentation,
	which is used in aircraft navigation and can result in a precision of 2m. The
	accuracy of consumer equipment is more like 10m on average, but it can be
	signiﬁcantly worse, for several reasons.

First, if the visible satellites are clustered close together this dilutes the

precision, which may happen if only a few satellites are visible. In this case,
	you can look up the resulting dilution of precision and use it to estimate the
	error. (For the key ﬁxes in our ﬁrst case, only ﬁve satellites were visible and
	the expected error was 45m; there are websites where you can look this up
	as a function of location and time.)	Second, many consumer devices (such

as phones) have snap-to-ﬁt software that automatically places the device on
	the nearest road or path. Third, even larger errors can come from multipath
	– typically when a signal reﬂected from a building competes with the direct
	signal. The combination of multipath and snap-to-ﬁt is what causes your phone
	or your navigator to jump from one street to another when driving or walking
	through a town with tall buildings. Finally, there are various kinds of jamming,
	ranging from barrage jamming that simply denies service to more sophisticated
	strategies such as meaconing in which a decoy retransmits the radio spectrum
	observed at another location, causing GPS equipment to believe it is at that
	location instead. Until recently, GPS jamming was something governments did,
	but the advent of low-cost software radios is starting to spread the fun. If I were
	a gangster on tag, I could use meaconing to provide an alibi: it would tell the
	police I’d been at home while I’d actually gone out and shot someone.

If you’re going to base a business on GPS, whether directly or by relying on

an underlying mapping service, it’s a good idea to understand not just the av-
	erage error but the worst case, and the circumstances in which such outliers can
	arise6. It’s possible to do better than commodity equipment, whether by using
	professional equipment or by using clever signal processing. One of my post-
	docs, Ramsey Faragher, did a startup (Focal Point Positioning) which applies
	interferometry to successive GPS ﬁxes in order to increase accuracy and detect
	both multipath and many kinds of jamming, supporting precision of about one

6You should employ at least one engineer who’s read up on the subject (such as via [1017])

and follows the relevant blogs (such as https://www.insidegnss.com.

| Security Engineering | 479 | Ross Anderson |
| --- | --- | --- |

14.5. POSTAGE METERS

metre7.

At the organisational level, the court cases gave insight into how the tech-

nology was working its way into police practice. A signiﬁcant proportion of
	burglaries are committed by ‘proliﬁc persistent o↵enders’ – typically men with
	drug and alcohol problems with dozens of convictions for minor o↵ences. (Our
	ﬁrst case was of a man alleged to have snuck into someone’s kitchen and stolen
	a bottle of wine from the fridge.) If police ﬁt their ‘frequent ﬂiers’ with curfew
	tags, then when a burglary was reported they can simply look up to see if any
	of them had been within 100 yards, and if so send a car to pick them up. This
	may help the police drive down the crime statistics by locking up the frequent
	ﬂiers for ever longer sentences; it might be less optimal socially if it ﬁlls up the
	jails with men who should be on rehab or receiving psychiatric care – or if it
	diverts attention from the more capable o↵enders.

14.5	Postage Meters

My fourth case history of metering is the postage meter. Postage stamps were
	introduced in Britain 1840 by Sir Rowland Hill to simplify charging for post,
	and developed into a special currency that could be used for certain purposes,
	from paying for postage to paying certain taxes and topping up the value of
	postal money orders. Bulk users of the postal system started to ﬁnd stamps
	unsatisfactory by the late 19th century, and the postage meter was invented in
	1889 by Josef Baumann. Its ﬁrst commercial use was in Norway in 1903; in the
	USA Arthur Pitney and Walter Bowes had a meter approved for use in 1920
	and built a large business on it. Early postal meters were analogue, and would
	print a stamp (known as an indicium) on a letter, or on a tape to stick on a
	parcel. The indicium had a date so that old indicia couldn’t be peeled o↵ and
	reused. Each meter had a mechanical value counter, protected by a physical
	seal; every so often you’d take your meter into the post o�ce to be read and
	reset. Fraud prevention relied on users taking their mail to the local post o�ce,
	which knew them; the clerk could check the date and the meter serial number.

In 1979, Pitney Bowes introduced a ‘reset-by-phone’ service, which enabled

ﬁrms to buy an extra $500 worth of credit over the phone; the implementation
	involved a mechanical one-time pad, with the meter containing a tape with
	successive recharge codes [477]. In 1981, this was upgraded to a DES-based
	system that enabled a meter to be recharged with any sum of money.	The

recharge codes were calculated in part from the value counter – so if the ﬁrm
	lied about how much postage they’d used, they couldn’t recharge the device.
	However, these meters still produced inked indicia.

In 1990, Jos´e Pastor of Pitney Bowes suggested replacing stamps and indicia

with printed digital signatures [1497]. This caught the attention of the US Postal
	Service, which started a program to investigate whether cryptography could help
	produce better postage meters. One concern was whether the availability of color
	scanners and copiers would make stamps and indicia too easy to forge. A threat
	analysis done for them by Doug Tygar, Bennett Yee and Nevin Heintze revealed
	that the big problem was not so much the forging or copying of stamps, or even

7Full disclosure: I invested in the company.

| Security Engineering | 480 | Ross Anderson |
| --- | --- | --- |

14.5. POSTAGE METERS

tampering with meters to get extra postage. It was bulk mailers corrupting
	Postal Service employees so as to insert truckloads of junk mail into the system
	without paying for them [1912].	As a bulk mailer on the ﬁddle would risk

arousing the suspicion of postal sta↵, there was a temptation to cut them in
	on the deal; and then it was natural to forge a meter plate whose inducting
	post o�ce was elsewhere. By 1990 US Postal service losses were in nine ﬁgures,
	and through the 1990s there were a number of high-proﬁle convictions of bulk
	mailers who had manipulated their meters, getting away with millions of dollars
	of free postage [265].

This led to a development programme for a meter using digital signatures,

generated by tamper-resistant processors in the postage meters. This was de-
	veloped into an open standard available to multiple competing manufacturers.
	The basic idea is that the indicium, which is machine-readable, contains both
	the sender and recipient postal codes, the meter number, the date, the postage
	rate, the amount of postage ever sold by the meter and the amount of credit
	remaining in it, all protected with a digital signature. The private signature
	key is kept in the meter’s processor while its corresponding public signature
	veriﬁcation key is kept in a Postal Service directory, indexed by the meter se-
	rial number. In this way, postal inspectors can sample mail in bulk at sorting
	o�ces, checking that each item is not only franked but on a logical route from
	its ostensible source to its destination.

The USA introduced the technology in 2000, with traditional suppliers such

as Pitney Bowes selling traditional meters while startups such as stamps.com
	obtained licenses to generate indicia online so that customers could download
	them and print them on their computers at home. Germany and the UK were
	next in 2004 and Canada in 2006; other countries followed suit. By 2006, all
	US postal facilities had the scanners needed to read the new indicia, of which
	an example is illustrated in Figure 14.4 below.

Town Circle or Postmark

Licensing Post Office/Mailed From ZIP Code

| FIM Mark | Date of Mailing (as required) |
| --- | --- |

Postage Amount

| optional advertising | MERRIFIELD VA 22180 | $0.32 | Rate Category |
| --- | --- | --- | --- |
|  | March 31, 1998 | FIRST-CLASS |  |
| art area |  |  |  |

US  POSTAGE	DEVICE 12345678901234

Device ID

Two Dimensional Barcode

| •Licensing ZIP Code •Destination ZIP Code •Software ID •Ascending Register •Descending Register | •Signature Algorithm Flag•Digital Signature •Device ID •Rate Category •Date of Mailing •Reserve Field •Postage •Version |
| --- | --- |

Figure 14.4: – one of the new formats for US postal meters (courtesy of Symbol
	Technologies)

Such indicia can be produced by postage meters that are drop-in replace-

| Security Engineering | 481 | Ross Anderson |
| --- | --- | --- |

14.5. POSTAGE METERS

ments for the old-fashioned devices; you weigh a letter, frank it, and get billed at
	the end of the month. You don’t have to take the meter in to be read though, as
	that can be done over the Internet for a credit meter, while if you buy a prepay-
	ment meter you replenish it by phoning a call centre and buying a magic number
	with your credit card. This works in much the same way as the prepayment
	electricity meters discussed earlier in this chapter.

Indicia can also be bought over the Internet by simply specifying the sender

and destination postal codes. This ‘online postage’ is aimed at small ﬁrms and
	people working from home who don’t send enough mail for it to be worth their
	while buying a meter. Both metered and online postage are cheaper than stamps
	to distribute. It has also become possible to manage the system much better,
	by tracking volumes and proﬁtability of mail down to local level. So, all told,
	digital post o↵ers more ﬂexibility to both users and postal services. But what
	about security?

Postage meters are a slight extension of the utility metering model. There’s

a tamper-resistant processor, either in the meter itself, or attached to a web
	server in the case of online postage; this has a value counter and a crypto key.
	It dispenses value by creating indicia until the value counter is exhausted, then
	requires replenishment from a control unit higher up in the chain. There are
	some additional features in each case. Many postage meters include a ‘Clark-
	Wilson’ feature whereby the value counter actually consists of two counters, an
	Ascending Register (AR) containing the total value ever dispensed by the meter,
	and a Descending Register (DR) indicating the remaining credit. The balancing
	control is AR + DR = TS, the ‘total setting’, that is, the total of all the sales
	made by or authorised for that device. If the balance fails, the meter locks up
	and can only be accessed by inspectors.

The full threat model includes stolen postage meters, meters that have been

tampered with to provide free postage, genuine meters used by unauthorised
	people, mail pieces with indicia of insu�cient value to cover the weight and
	service class, and straightforward copies of valid indicia. Various sampling and
	other tests are used to control these risks. Subtleties include how you deal with
	features like certiﬁed mail and reply mail. There are also national di↵erences
	on matters ranging from which authentication algorithms are used to what sort
	of usage data the meters have to upload back to the postal service.

Once operators got real experience, the industry started to move away from

digital signatures to message authentication codes. Signatures appealed because
	they were elegant; but in real life, signature veriﬁcation is expensive, and has
	also turned out to be unnecessary. Equipment at major sorting o�ces must
	process thousands of mail pieces a minute, and postal services usually verify
	indicia as an o✏ine batch operation. Forged mail pieces go through initially
	and are only intercepted once a pattern of abuse emerges. Once veriﬁcation is
	centralised, MACs make more sense than signatures; the central servers have
	hardware security modules with master keys that are diversiﬁed to a MAC key
	in each meter, just as with utility meters. It turns out that two-digit MACs are
	enough to detect systematic abuse before it becomes signiﬁcant [477].

In many countries, the postal service contracts all the cryptography out to

the meter vendors. So indicia are veriﬁed only in the home postal system, as

| Security Engineering | 482 | Ross Anderson |
| --- | --- | --- |

14.6. SUMMARY

overseas systems will often use di↵erent vendors.	We also see a diversity of

architectures.	Canada, for example, uses both signatures and MACs on its

indicia. (And if you want to bribe a postal employee to let a few tons of junk
	mail into the system, the place to do it is now at a border crossing.)

How stu↵ actually breaks in real life is – as always – instructive. In the

German post o�ce’s ‘Stampit’ scheme, a user buys ‘smart pdf’ ﬁles that contact
	the post o�ce to say they’re being printed, without any interaction with the
	user or her software. If the paper jams, or the printer is out of toner, then
	tough. So users arrange to photocopy the stamp, or to copy it to a ﬁle from
	which it can be printed again if need be. The UK system has learnt from this:
	although a stamp is grey-listed when a user PC reports that it’s been printed,
	the grey doesn’t turn to black until the stamp appears at the sorting o�ce. The
	di↵erence in syntax is subtle: the German system tried to stop you printing the
	stamp more than once, while the British system more realistically tries to stop
	you using it more than once [884].

All told, moving to digital postal meters enabled much better control than

was possible in the old days, when postal inspectors had to refer to paper records
	of mechanical meter readings. It also facilitates business models that extend
	the service to many more customers and that also improve the post o�ce’s cash
	ﬂow and credit control. Unlike digital tachographs, digital postal meters have
	brought real beneﬁts.

14.6	Summary

Many security systems are concerned one way or another with monitoring or
	metering some aspect of the environment. They range from utility meters to
	taxi meters, tachographs, and postal meters. We’ll come across further metering
	and payment systems in later chapters, such as the mechanisms used to stop
	printer cartridges working once they have printed a certain number of pages.

Many monitoring, metering and payment systems have been redesigned as

the world moved from analogue to digital technology. Some of the redesigns
	have been a success, and others less so. Digital prepayment electricity meters
	have been a success, as they enable utility companies in the developing world
	to sell power to hundreds of millions of people who don’t even have addresses,
	let alone credit ratings. Digital tachographs have been less impressive; they just
	do what the old analogue systems did, but less well. Their slow evolution was
	perhaps inevitable given the many entrenched stakeholders and the lack of op-
	portunity for a disruptive process change, as the goal was securing compliance
	by a mature industry with existing safety law. Our third example, the curfew
	tag, extends location monitoring from vehicles to human beings. It has sup-
	ported some innovation, since technical o↵ender monitoring is a new industry;
	it also teaches us some of the limits of using GPS in large complex systems. Our
	fourth example, the postage meter, did allow some competitive innovation and
	has been a success.

As with burglar alarms, the protection of monitoring systems is tied up with

dependability. You have to think long and hard about what sort of service denial

| Security Engineering | 483 | Ross Anderson |
| --- | --- | --- |

14.6. SUMMARY

attacks are possible. Key management can be an issue, especially in low-cost
	widely-distributed systems where you can’t provide a central key management
	facility or hire enough trustworthy people.	Systems may have to deal with

numerous mutually suspicious parties, and must often be implemented on the
	cheapest possible hardware. Many of the monitoring devices are in the hands
	of opponents. And there are all sorts of application-level subtleties that have to
	be understood if you want your design to succeed.

Research Problems

There’s a lot of talk about the ‘Internet of Things’ but few concrete examples for
	researchers to think about. Case studies such as those described here may help.
	Although the mechanisms (and products) developed for payment networks can
	be adapted (and are), much of the design work has to be redone and the end
	result often has vulnerabilities. Metering applications are particularly useful
	because of the pervasive mutual mistrust caused not just by competing com-
	mercial entities but by the presence of dishonest sta↵ at every level, as well as
	dishonest customers; and the fact that most of the equipment is in the custody
	of the attackers.

Again, there are questions for security economists and scholars of innovation.

Why did some digital transformations of existing metering systems work well
	(utilities, postage) while others were less impressive (tachographs)? Why were
	some disruptive, in that new entrants successfully challenged the previous in-
	cumbent suppliers, while in other cases (such as postage) the existing suppliers
	managed the transition to better digital systems and survived despite innovative
	competition from dotcom startups?

Further Reading

Prepayment electricity meters are described in [93]. Tachographs are written
	up in [64]; other papers relevant to transport appear in the annual ESCAR
	conference on electronic security in cars. The early work on postal meters is
	in [1912] and the US regulations can be found in [1320]. However the most
	detailed exposition of postage meter security is a book by Gerrit Bleumer of
	Francotyp-Postalia, which played a leading role in the program [265].

| Security Engineering | 484 | Ross Anderson |
| --- | --- | --- |